模拟运行的项目代码如下：

#include <opencv2/opencv.hpp>

#include <iostream>

#include <vector>

#include <cmath>

#include <thread>

using namespace cv;

using namespace std;

vector<Point2f> selectedPoints;

vector<vector<Point2f>> allObjectPoints;

void onMouse(int event, int x, int y, int flags, void* userdata) {

if (event == EVENT_LBUTTONDOWN && selectedPoints.size() < 7) {

selectedPoints.push_back(Point2f(x, y));

cout << "选中角点 " << selectedPoints.size() << ": (" << x << ", " << y << ")" << endl;

Mat* img = (Mat*)userdata;

circle(*img, Point(x, y), 6, Scalar(0, 255, 0), -1);

putText(*img, to_string(selectedPoints.size()), Point(x + 8, y - 8),

FONT_HERSHEY_SIMPLEX, 0.6, Scalar(0, 255, 0), 2);

imshow("选择角点", *img);

}

}

void createExampleCalibFile(const string& filename) {

FileStorage fs(filename, FileStorage::WRITE);

if (fs.isOpened()) {

Mat cameraMatrix = (Mat_<double>(3, 3) << 800, 0, 320, 0, 800, 240, 0, 0, 1);

Mat distCoeffs = (Mat_<double>(1, 4) << 0.1, -0.05, 0, 0);

fs << "cameraMatrix" << cameraMatrix << "distCoeffs" << distCoeffs;

fs.release();

cout << "[INFO] 已生成示例标定文件" << endl;

}

}

bool loadCalibration(const string& filename, Mat& cameraMatrix, Mat& distCoeffs) {

FileStorage fs(filename, FileStorage::READ);

if (!fs.isOpened()) {

cout << "[ERROR] 无法加载标定文件，使用默认参数" << endl;

cameraMatrix = (Mat_<double>(3, 3) << 800, 0, 320, 0, 800, 240, 0, 0, 1);

distCoeffs = (Mat_<double>(1, 4) << 0.1, -0.05, 0, 0);

return true;

}

fs["cameraMatrix"] >> cameraMatrix;

fs["distCoeffs"] >> distCoeffs;

fs.release();

return true;

}

struct Material {

string name;

Point3f safeCenter;

float safeRadius;

Point3f currentPos;

bool isAlarm;

bool isAuthorized;

int consecutiveOutFrames;

int consecutiveInFrames;

Material(const string& n, float x, float y, float z, float r)

: name(n), safeCenter(x, y, z), safeRadius(r),

currentPos(0,0,0), isAlarm(false), isAuthorized(false),

consecutiveOutFrames(0), consecutiveInFrames(0) {}

void updatePosition(float x, float y, float z) {

currentPos = Point3f(x, y, z);

float dx = x - safeCenter.x;

float dy = y - safeCenter.y;

float dz = z - safeCenter.z;

float distance = sqrt(dx*dx + dy*dy + dz*dz);

if (distance > safeRadius) {

consecutiveOutFrames++;

consecutiveInFrames = 0;

if (consecutiveOutFrames >= 5 && !isAuthorized) {

if (!isAlarm) {

isAlarm = true;

cout << "[ALARM] " << name << " 位置偏移! 距离: " << distance << "m" << endl;

}

}

} else {

consecutiveInFrames++;

consecutiveOutFrames = 0;

if (consecutiveInFrames >= 10) {

if (isAlarm) {

isAlarm = false;

cout << "[INFO] " << name << " 返回安全区域，报警取消" << endl;

}

}

}

}

void authorizeTake() {

isAuthorized = true;

isAlarm = false;

consecutiveOutFrames = 0;

consecutiveInFrames = 0;

cout << "[INFO] " << name << " 授权取餐成功，不再报警" << endl;

}

};

vector<Material> materials;

// 简单的标志位

bool g_running = true;

int main() {

cout << "=== 物资位置监控系统 ===" << endl;

cout << "控制说明：" << endl;

cout << "  在图像窗口按 'q' 退出" << endl;

cout << "  在图像窗口按 'a' 授权取餐" << endl;

cout << "  选点时按 ESC 退出" << endl;

// 先打开摄像头！

VideoCapture cap(0);

if (!cap.isOpened()) {

cout << "[ERROR] 无法打开摄像头" << endl;

return -1;

}

cap.set(CAP_PROP_FRAME_WIDTH, 640);

cap.set(CAP_PROP_FRAME_HEIGHT, 480);

cout << "[INFO] 摄像头已打开: 640x480" << endl;

// 加载标定参数

Mat cameraMatrix, distCoeffs;

loadCalibration("calibSettings.xml", cameraMatrix, distCoeffs);

cout << "[INFO] 相机标定参数加载完成" << endl;

// 输入物体数量

cout << "请输入要跟踪的物体数量: ";

int objectCount;

cin >> objectCount;

cin.ignore(); // 清除换行符

// 初始化物资

materials.emplace_back("外卖A", 0.0f, 0.0f, 0.0f, 0.15f);

cout << "[INFO] 初始化物资: " << materials[0].name << " 安全半径: " << materials[0].safeRadius << "m" << endl;

// 手动选点

vector<Point3f> modelPoints = {

Point3f(-0.1f, -0.1f, 0.0f), Point3f(0.1f, -0.1f, 0.0f),

Point3f(-0.1f,  0.1f, 0.0f), Point3f(-0.1f, -0.1f, 0.2f),

Point3f(0.1f, -0.1f, 0.2f), Point3f(0.1f,  0.1f, 0.2f),

Point3f(-0.1f,  0.1f, 0.2f)

};

for (int i = 0; i < min(objectCount, 1); ++i) {

cout << "\n=== 请选择物体的 7 个角点 ===" << endl;

cout << "顺序：底面(前左→前右→后左) + 顶面(前左→前右→后右→后左)" << endl;

Mat frame;

cap >> frame;

if (frame.empty()) {

cout << "[ERROR] 无法获取图像" << endl;

break;

}

selectedPoints.clear();

namedWindow("选择角点", WINDOW_AUTOSIZE);

setMouseCallback("选择角点", onMouse, &frame);

// 选点循环

while (selectedPoints.size() < 7 && g_running) {

imshow("选择角点", frame);

int key = waitKey(30) & 0xFF;

if (key == 27) { // ESC键

destroyWindow("选择角点");

g_running = false;

break;

}

}

destroyWindow("选择角点");

if (!g_running) break;

allObjectPoints.push_back(selectedPoints);

cout << "角点选择完成，共 " << selectedPoints.size() << " 个点" << endl;

}

if (!g_running) {

cap.release();

return 0;

}

// 计算初始位姿

if (allObjectPoints.size() > 0) {

vector<Point3f> modelPoints3D = {

Point3f(-0.1f, -0.1f, 0.0f), Point3f(0.1f, -0.1f, 0.0f),

Point3f(-0.1f,  0.1f, 0.0f), Point3f(-0.1f, -0.1f, 0.2f),

Point3f(0.1f, -0.1f, 0.2f), Point3f(0.1f,  0.1f, 0.2f),

Point3f(-0.1f,  0.1f, 0.2f)

};

Mat rvec, tvec;

bool success = solvePnP(modelPoints3D, allObjectPoints[0], cameraMatrix, distCoeffs, rvec, tvec);

if (success) {

cout << "\n=== 开始实时监控 ===" << endl;

Mat rvec_f, tvec_f;

rvec.convertTo(rvec_f, CV_32F);

tvec.convertTo(tvec_f, CV_32F);

// 预计算固定定位框

vector<Point3f> cubeEdges3D = {

Point3f(-0.1f, -0.1f, 0.0f), Point3f(0.1f, -0.1f, 0.0f),

Point3f(0.1f, -0.1f, 0.0f), Point3f(0.1f,  0.1f, 0.0f),

Point3f(0.1f,  0.1f, 0.0f), Point3f(-0.1f,  0.1f, 0.0f),

Point3f(-0.1f,  0.1f, 0.0f), Point3f(-0.1f, -0.1f, 0.0f),

Point3f(-0.1f, -0.1f, 0.2f), Point3f(0.1f, -0.1f, 0.2f),

Point3f(0.1f, -0.1f, 0.2f), Point3f(0.1f,  0.1f, 0.2f),

Point3f(0.1f,  0.1f, 0.2f), Point3f(-0.1f,  0.1f, 0.2f),

Point3f(-0.1f,  0.1f, 0.2f), Point3f(-0.1f, -0.1f, 0.2f),

Point3f(-0.1f, -0.1f, 0.0f), Point3f(-0.1f, -0.1f, 0.2f),

Point3f(0.1f, -0.1f, 0.0f), Point3f(0.1f, -0.1f, 0.2f),

Point3f(0.1f,  0.1f, 0.0f), Point3f(0.1f,  0.1f, 0.2f),

Point3f(-0.1f,  0.1f, 0.0f), Point3f(-0.1f,  0.1f, 0.2f)

};

vector<Point2f> projected;

projectPoints(cubeEdges3D, rvec_f, tvec_f, cameraMatrix, distCoeffs, projected);

// 主监控循环

int frameCount = 0;

int inZoneFrames = 0;

int outZoneFrames = 0;

while (g_running) {

// 第一优先级：检测按键

int key = waitKey(30) & 0xFF;

if (key == 'q' || key == 'Q') {

cout << "\n[INFO] 收到退出指令，程序结束" << endl;

g_running = false;

break;

}

if (key == 'a' || key == 'A') {

materials[0].authorizeTake();

}

// 获取图像

Mat frame;

cap >> frame;

if (frame.empty()) {

cout << "[ERROR] 无法获取图像" << endl;

break;

}

// 模拟位置变化

float simX = 0.0f, simY = 0.0f;

if (frameCount > 100 && frameCount <= 200) {

simX = 0.25f;  // 超出安全半径

simY = 0.1f;

outZoneFrames++;

inZoneFrames = 0;

} else if (frameCount > 200) {

simX = 0.05f;  // 返回安全区域

simY = 0.05f;

inZoneFrames++;

outZoneFrames = 0;

} else {

inZoneFrames++;

outZoneFrames = 0;

}

// 更新位置检测

materials[0].updatePosition(simX, simY, 0.0f);

// 显示状态

string status = materials[0].isAlarm ? "ALARM!" : "NORMAL";

Scalar statusColor = materials[0].isAlarm ? Scalar(0, 0, 255) : Scalar(0, 255, 0);

string authStatus = materials[0].isAuthorized ? " [AUTHORIZED]" : "";

putText(frame, "Object: " + materials[0].name + " - " + status + authStatus,

Point(10, 30), FONT_HERSHEY_SIMPLEX, 0.6, statusColor, 2);

string zoneInfo = "In:" + to_string(inZoneFrames) + " Out:" + to_string(outZoneFrames);

putText(frame, zoneInfo, Point(10, 60), FONT_HERSHEY_SIMPLEX, 0.5, Scalar(255, 255, 255), 1);

string posInfo = "Pos: (" + to_string(simX).substr(0, 4) + ", " + to_string(simY).substr(0, 4) + ")";

putText(frame, posInfo, Point(10, 90), FONT_HERSHEY_SIMPLEX, 0.5, Scalar(255, 255, 255), 1);

// 绘制固定定位框

for (size_t i = 0; i < projected.size(); i += 2) {

line(frame, projected[i], projected[i + 1], Scalar(0, 255, 0), 2);

}

// 报警时显示警告

if (materials[0].isAlarm) {

rectangle(frame, Rect(10, 120, 250, 60), Scalar(0, 0, 255), 2);

putText(frame, "OUT OF SAFE ZONE!", Point(15, 140),

FONT_HERSHEY_SIMPLEX, 0.5, Scalar(0, 0, 255), 1);

} else if (inZoneFrames > 0) {

putText(frame, "SAFE ZONE", Point(10, 120),

FONT_HERSHEY_SIMPLEX, 0.5, Scalar(0, 255, 0), 1);

}

imshow("物资位置监控系统", frame);

frameCount++;

this_thread::sleep_for(chrono::milliseconds(50));

}

cout << "[INFO] 主循环结束" << endl;

} else {

cout << "[ERROR] solvePnP 计算失败" << endl;

}

}

// 清理资源

g_running = false;

cap.release();

destroyAllWindows();

cout << "[INFO] 程序正常退出" << endl;

return 0;

}